Significant changes, for Shippers, with the new IMDG Code Amendment 36-12

IMDG Code Amendment 36-12 may be used voluntary from January 1, 2013 (subject to national administration adoption), and will be mandatory effective January 1, 2014.

UN NUMBERS

- Ten new UN numbers are now included in 36-12 IMDG Code as shown below.

UN3497 KRILL MEAL

UN3498 IODINE MONOCHLORIDE, LIQUID

UN3499 CAPACITOR, electric double layer (with an energy storage capacity greater than 0.3Wh)

UN3500 CHEMICL UNDER PRESSURE,N.O.S.

UN3501 CHEMICL UNDER PRESSURE,FLAMMABLE, N.O.S.

UN3502 CHEMICL UNDER PRESSURE,TOXIC, N.O.S.

UN3503 CHEMICL UNDER PRESSURE,CORROSIVE, N.O.S.

UN3504 CHEMICL UNDER PRESSURE,FLAMMABLE,TOXIC, N.O.S.

UN3505 CHEMICL UNDER PRESSURE,FLAMMABLE, CORROSIVE, N.O.S.

UN3506 MERCURY CONTAINED IN MANUFACTURED ARTICLES

LIMITED QUANTITIES

- Class 1.4S explosives have three (3) UN numbers that may be consigned as limited quantities, but are still subject to the rules of section 4.1.5.

- Class 8 packing group II (in limited quantities), for liquids in glass or similar inners also need rigid intermediate packaging.

- Other substances in limited quantities in fragile inners will need suitable intermediate packaging when in shrink-wrapped trays.

STOWAGE & SEGREGATION

- Chapter 7.3 is now about loading goods into Cargo Transport Units (Containers).

FOODSTUFFS

- Segregation from foodstuffs (which is now defined in 1.2.1) has changed. 'Away from' foodstuffs and ‘Separated from’ foodstuffs are no longer mentioned.

Generally, a class or sub-risk of 2.3, 6.1, 6.2, 7 and 8, plus a few specific entries in the Dangerous Goods List, shall not be in the same Cargo Transport Unit as foodstuffs. However, some class 6.1 or 8 items, plus a few specific entries in the Dangerous Goods List, will be allowed in the same Cargo Transport Unit as food, provided they are at least 3 meters apart, without needing competent authority approval.

LITHIUM BATTERIES

- Section 2.9.4 covers further defines lithium batteries requirements and how they should be classified.